.PL

WHATS-UP.DOC Release 0.55 Page 1.

_______

____|__     |                  (tm)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

WHATS-UP (c) Joe Kasser, G3ZCZ, 1990

Joe Kasser G3ZCZ/W3

POB 3419

Silver Spring, Md. 20918

(301) 593 6136

G3ZCZ @ N4QQ.MD.USA

This  version of WHATS-UP allows you to capture, decode,  display

and extract for analysis telemetry from the Digital Voice Encoder

(DOVE) OSCAR 17, Fuji-OSCAR 20 spacecraft.  It also allows you to

process   captured   telemetry  from  the  late   Fuji-OSCAR   12

spacecraft.    WHATS-UP  is  a  table  driven  program  via   the

configuration  files to allow maximum flexibility.  This  program

does  not decode/display the AMSAT/UoSAT Binary  telemetry  data,

and should not be used to capture binary telemetry.

The program is distributed as a Shareware product. You may freely

copy  and  share  the product for noncommercial  use,  with  your

friends,  associates and other radio hams. If you decide  to  use

the  product, you are asked to become a registered user  by  com-

pleting  the  registration  form and sending it,  and  $35.00  or

equivalent in foreign currency to the author.

Upon  receipt  of your registration, you will  receive  one  free

update disk, telephone and mail (electronic and regular) support.

This product may not be sold or distributed with another  product

without the express written permission of Joe Kasser, G3ZCZ.

Joe  Kasser,  G3ZCZ will only support unmodified copies  of  this

software.  Your comments and suggestions for changes are  however

welcome.  If you are the first to suggest a change that is imple-

mented,  you will be sent a complimentary copy of the  disk  with

the change incorporated.

Potential  Commercial  and Educational Institution  Users  please

contact  Joe Kasser directly for modifications and/or details  of

Site licensing.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 2.

Table of Contents

1.0 Introduction

2.0 Capabilities

3.0 Things You Should Know

3.1 TNC State

3.2 Limit Checking

3.3 Link Quality Measurements

3.4 Program Requirements

3.5 Data Format

3.6 Display Pages

3.7 Function Keys

3.8 Typical Screen Color Combinations

3.9 TNCs

3.10 Obtaining Updates

3.11 Look Up Tables

4.0 Bringing Up WHATS-UP For the First Time

4.1 Editing the Configuration File.

4.2 Starting the program.

5.0 Standby Mode

5.1 Extract From Playback File

5.2 Interactive Mode

5.3 Playback Mode

5.4 AMSAT/OSCAR Menu

5.5 Real Time Mode

5.6 Change Directory Path

5.7 Change Display Page

5.8 Show Space on Disk

5.9 Change Playback File

5.10 Change Microsat

5.11 Show Defaults

5.12 View Playback File

5.13 Exit to Dos

5.14 Show Files

5.15 Show Color Chart

6.0 Playback Mode

7.0 Interactive Mode

8.0 Real Time Mode

9.0 (Data) Extraction Mode

10.0 Configuration File

11.0 Spacecraft Parameter Files

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 3.

12.0 Telemetry Channel Extraction File

13.0 Extracted Telemetry Data File

APPENDIX A DOVE Telemetry

APPENDIX B Fuji-OSCAR 20 Telemetry

APPENDIX C Fuji-OSCAR 12 Telemetry

APPENDIX D Information about AMSAT

APPENDIX E Amateur Radio Software by Joe Kasser G3ZCZ

APPENDIX F Shareware

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 4.

1.0 Introduction

Orbiting  Satellites  Carrying Amateur Radio  (OSCAR)  send  back

volumes of Telemetry daily and apart from a few Command  stations

no-one seems to be doing anything with it.  We, as Radio Amateurs

tend  to  concentrate on the communications capabilities  of  the

spacecraft  and  ignore  their telemetry completely.   If  we  do

listen  to  a  beacon,   it's usually  just  to  check  that  the

transponder is on, heaven forbid - to actually copy any data.

The  telemetry  can  tell us a story.  It can  tell  us  what  is

happening to both the spacecraft and its environment.  As such it

has  a tremendous educational potential which has  remained  just

that - a potential for at least the last six years.

Before  every satellite launch the equations and format  for  the

spacecraft telemetry are published by AMSAT.  The telemetry tells

us  about  the health and welfare of the spacecraft  itself,  and

something  about  the  payload.  Spacecraft  health  and  welfare

information tells us about the battery, solar cells and on  board

computer  status. Payload information can range from  information

about  transponder loading/utilization to data  from  instruments

that  measure  the  environment of the space in  and  around  the

satellite.  Battery Telemetry is used by the command stations  to

determine  when  the  spacecraft  can  be  used,  and  when   the

transponders should be shut down.  The number of individuals  not

associated  with  command stations who  have  decoded  spacecraft

telemetry  and published their findings can be counted using  the

fingers of one hand.  There's a lot of computing power out  there

that  has  the  potential  to  process  telemetry  and   discover

something new, but does not have the access to the data.   WHATS-

UP  is  an  attempt to provide that data  to  the  average  Radio

Amateur.

2.0 Capabilities

WHATS-UP contains the following features:

* Real-time, Interactive and Playback modes.

* Automatic Capture-to-disk of raw telemetry.

* Extracts  telemetry channel data to a database  or  spreadsheet

readable file for further analysis.

* Link quality measurement.

* Capability  to  display and print the raw telemetry  as  it  is

received.

* Up to 16 user configurable display pages (screens). You set the

position  on  the page (width of engineering  unit  field,  and

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 5.

number of decimal places) that a parameter is displayed at.

* Wild card page (parameter shows up on all pages).

* Selectable  display of Engineering units or Hex byte  for  each

display page.

* Display of raw packets (i.e. STATUS)

* Color  changes if a parameter value changed between  successive

frames.

* Audio  and  visual alarms if a telemetry value  exceeds,  falls

below or falls outside a preset limit value(s).

* Dumb split screen terminal mode (a la LAN-LINK).

* Customizable colors, PC to TNC baud rate, data parity and  stop

bits.

* Default spacecraft configuration files.

* Time of day clock display (in HH:MM:SS format)

3.0 Things You Should Know

3.1 TNC State

WHATS-UP  does not change the TNC state at start up,  unless

you command a time change or a configuration.

3.2 Limit Checking

Limits  are only checked for parameters being displayed  (in

Engineering  units).  With  this approach, you  can  set  up

different  pages for different on-board subsystems, you  can

also  set up different display pages of the same  parameters

for daylight, darkness and terminator crossing passes,  with

different limit values to draw your attention to changes.

3.3 Link Quality Measurements

The  link quality part allows the following to be done.  You

can  define which of the packets you want  to  display/count

(If  you just want to count them and not display  them,  set

the  page  value to 99). You can then view  an  incrementing

count  counter each time a selected packet is received.  For

example,  using  DOVE-OSCAR  17, you can  also  display  the

contents  of  the  STATUS, WASH or  BCRXMT  packets  in  any

display page.  This feature allows you to get an idea of how

good your receiving system is.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 6.

3.4 Program Requirements

IBM PC or clone with 256k memory.

A  Packet TNC with an RS-232 interface is required for  real

time data capture.  WHATS-UP has been tested with a TNC2, an

MFJ1278, a KAM and a PK-232 so far.

3.5 Data Format

The program DOES require that the telemetry be received, and

captured-to-disk with the packet header on a different  line

to  the  contents  of the packet  (HEADERLINE  ON).  I  also

suggest  that  you turn the date/time stamp on so  that  you

will  be  able to playback your data  and  extract  selected

values  and their corresponding time codes into a file  that

can be read into your spreadsheet program for further  trend

analysis.

3.6 Display Pages

This version (the default) is set up to display all  packets

as  wild  cards (i.e will show up on all  pages),  and  then

display several temperatures and solar cell array currents.

By  putting  the correct parameters  in  the  SPACECRAFT.SYS

file, you can set up any of the pages to display any of  the

telemetry channel data in any row and column in that page.

3.7 Function Keys

The following function keys are active:

FK 1 capture to disk Toggle

FK 2 type of display Engineering Units/Raw Byte Toggle

FK 3 select display page

Alt-B send a 'Break' to the TNC (interactive mode only)

Alt-C connect  to another packet  station  (interactive

mode only)

Alt-D disconnect    from   another    packet    station

(interactive mode only)

Alt-P Printer on/off toggle

Alt-S Sound on/off toggle

Alt-X Quit Mode

left arrow  decreases playback speed

right arrow increases playback speed.

3.8 Typical Screen Color Combinations

You  can  view  the colors  associated  with  the  different

numbers by choosing the '*' option in the Main Menu.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 7.

3.9 TNCs

Version 0.55 is set up to configure the PK-232 in the AMSAT-

OSCAR  Mode.  You can use other TNCs in the packet modes  if

you configure them yourself.

3.10 Obtaining Updates

It  is  anticipated  that  WHATS-UP is  going  to  grow  and

incorporate  features for decoding and displaying data  from

other  spacecraft.   Microsat binary  telemetry  decode  and

display  capability will be added when AMSAT  announce  that

the  format has stabilized, and sufficient registered  users

express interest in having it.  To stay on the mailing  list

and receive an update as it is released, register your copy,

then  send  in a disk containing at least 300k  of  captured

data from the spacecraft of your choice.  If you would  like

to  exchange  data with other  educational  institutions  or

users so as to be able to analyze more data than you can get

on a single pass, indicate that fact and we will try and put

you in direct touch with others who are similarly inclined.

3.11 Look Up tables

WHATS-UP uses look up tables to determine how the  telemetry

is  decoded and where the decoded data are displayed.   This

allows  you  as  the user to  customize  display  pages  and

configure  new tables when changes are made in the data  the

spacecraft sends, or when new spacecraft are launched.   For

example,  a  look up table set for Fuji-OSCAR 12  will  also

work  with  fuji-OSCAR 20 if the equation  coefficients  and

description  text  are changed in  the  configuration  file.

Similar  configuration  files can be used for  each  of  the

AMSAT  Microsat ASCII telemetry data formats that were  used

in the months following the launch.

4.0 Bringing Up WHATS-UP For the First Time

WHATS-UP  has four basic modes of operation;  Interactive,  Real-

time,  Data Extraction and Playback. You must configure  WHATS-UP

before you try any Real-time activity.

4.1 Editing the Configuration File.

The configuration file is called WHATS-UP.SYS. You must edit

it  with an ASCII word processor (in the non document  mode)

to set up the correct parameters on the RS-232 link  between

your  TNC  and your PC. See Section 10 for details  of  what

parameter is on which line of the WHATS-UP.SYS file.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 8.

4.2 Starting the program.

You may start the program in three ways as follows.

4.2.1 Default

Type 'whats-up' and return (without the '  characters).

This brings the program up in the default mode. It will

read the whats-up.sys file to determine the  spacecraft

being monitored, and then prompt you for the mode.

4.2.2 User Chosen spacecraft

Type  'whats-up spacecraft' and return (without  the  '

characters). The program reads the spacecraft.sys  file

to load the parameters for the Microsat of choice,  and

then prompts you for the mode.

Examples of the command are :

'WHATS-UP DOVE' or 'WHATS-UP Fuji20'

4.2.3 Custom Mode

Type 'whats-up spacecraft mode' and return (without the

' characters). This brings the program up in the custom

mode.  It will read the whats-up.sys file to  determine

the  spacecraft being monitored, and then start  up  in

the  mode  you set. Valid modes are 'p', 'i',  'e'  and

'r'.

Examples of the command are :

'WHATS-UP DOVE R' or 'WHATS-UP Fuji20 R'

If  you  place  a  command  line  like  this  in   your

autoexec.bat  file, should you be copying telemetry  in

an  unattended manner and a power failure  take  place,

the system will boot up into the correct WHATS-UP  mode

when power is restored.

5.0 Standby Mode

The  standby Mode presents you with the Main Menu,  organized  in

two prompt windows, as shown below.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 9.

The MODES Menu Window

E Extract From Playback File

I Interactive Mode

P Playback Mode

O AMSAT/OSCAR Menu

R Real Time Mode

The SELECTIONS Menu Window

A Change Directory Path

C Change Display Page

D Show Space on Disk

F Change Playback File

M Change Microsat

S Show Defaults

V View Playback File

X Exit to Dos

Z Show Files

* Show Color Chart

Type the letter associated with the option to perform it.

Each of the options are described below.

5.1 Extract From Playback File

This option begins up the Extraction mode.

5.2 Interactive Mode

This option begins up the Interactive mode.

5.3 Playback Mode

This option begins up the Playback mode.

5.4 AMSAT/OSCAR Menu

When you exercise this option you will be presented with  the

Menu shown below.

A UoSAT ASCII Beacon

B Phase 3 RTTY Beacon

M Fuji/Microsat ASCII Packet

Each option is described below.

5.4.1 UoSAT ASCII Beacon

This option will configure the PK-232 to copy the UoSAT-OSCAR

11 telemetry. Note: you require a hardware change in the  PK-

232 to make sense of the received data.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 10.

5.4.2 Phase 3 RTTY Beacon

This option will configure the PK-232 to copy the UoSAT-OSCAR

13 Baudot Beacon.

5.4.3 Fuji/Microsat ASCII Packet

This  option will configure the PK-232 to copy the  DOVE  and

Fuji  ASCII  format  PACKET telemetry.  You  should  not  use

WHATS-UP  to  capture AMSAT/UoSAT  binary  telemetry  because

WHATS-UP filters the ^J and ^M (carriage return and line feed

characters) from the incoming datastream.

5.5 Real Time Mode

This option begins up the Real-time mode.

5.6 Change Directory Path

This  option allows you to temporarily change  the  directory

path.

5.7 Change Display Page

This  option  allows you to change the display page  for  the

Real-time,  Playback  and Extraction Modes. It  performs  the

same  operation as Function key 3 (F3) when those  modes  are

active.

5.8 Show Space on Disk

This  option  allows you to see how much space is left  on  a

disk with exiting from the program.

5.9 Change Playback File

This option allows you to change the playback file. To select

a file, move the cursor down to the desired file and push the

'Enter' key.  If you have more files than fit in the  window,

touch the 'PgDn' key to display another window full.

5.10 Change Microsat

This  option  allows  you to choose  another  spacecraft.  To

select  a another one, enter the name of  the  spacecraft.sys

file.   For example, the default files supplied with  Version

0.55  are  DOVE.SYS and FUJI.SYS. To select the DOVE  or  the

Fuji-20 spacecraft, type 'DOVE' or 'Fuji'

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 11.

5.11 Show Defaults

This option allows you to display the default settings.

5.12 View Playback File

This  option allows you to view the contents of the  playback

file.

5.13 Exit to Dos

This  option allows you to terminate WHATS-UP and  return  to

DOS.

5.15 Show Color Chart

This  option  allows you to display the  color  combinations.

Use  this  to see what how the different  color  combinations

appear on your screen, note the numbers associated with  each

color,  then exit from the program and edit the  WHATS-UP.SYS

file  using  your editor in its ASCII (non  document)mode  to

change the colors to those you desire.

6.0 Playback Mode

The Playback mode allows you to play back captured telemetry with

4  speeds (speedy, slow, slower and snail's pace).  If you  touch

the 'Escape' key you will bring up the following Menu.

D Show Space on Disk

R Reset Packet Counters

Q Quit to Main Menu

Z Show Spacecraft data Files

Type the letter associated with the option to perform it.

Each of the options are described below.

6.1 Show Space on Disk

This  option  allows you to see how much space is left  on  a

disk with exiting from the program.

6.2 Reset Packet Counters

This  option  resets the packet counters to  zero.  use  this

before  playing back a data file to see how many  packets  of

each type are present in the file.

6.3 Quit to Main Menu

This option returns you to the Main menu.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 12.

6.4 Show Spacecraft data Files

This  option  shows  you  the  data  files  for  the   chosen

spacecraft in the default directory path.

7.0 Interactive Mode

The  Interactive mode is a dumb terminal. You can use it to  give

commands  to the TNC. The usual one is to set the TNC  date  from

the  computer's  clock.  You  should  also  use  it  to  set  the

'HEADERLINE  ON'. In this mode, you will see the raw  packets  on

the channel. You can also use this mode as a regular TNC  program

(If you do, you ought to get your head examined, because LAN-LINK

will  do the job much better).  The capture-to-disk will turn  on

when  the first packet is copied, and will turn off  two  minutes

after the last.

If  you  touch the 'Escape' key you will bring up  the  following

Menu.

C Configure PK-232

D Show Space on Disk

O AMSAT/OSCAR Menu

Q Quit to Main Menu

Z Show Spacecraft data Files

Type the letter associated with the option to perform it.

Each of the options are described below.

7.1 Configure PK-232

This  option  configures the PK-232 to copy  the  UI  packets

transmitted by the Packet spacecraft.

7.2 Show Space on Disk

This  option  allows you to see how much space is left  on  a

disk with exiting from the program.

7.3 AMSAT/OSCAR Menu

This  option brings up the AMSAT/OSCAR Menu as  described  in

the Main Menu in the Standby mode.

7.4 Quit to Main Menu

This option returns you to the Main menu.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 13.

7.5 Show Spacecraft data Files

This  option  shows  you  the  data  files  for  the   chosen

spacecraft in the default directory path.

8.0 Real Time Mode

The  Real-time mode converts and displays engineering  data.  You

can  display up to 16 (configured by you) pages  of  information.

Information that changes between successive frames, is shown in a

different color. Information that has exceeded a preset (by  you)

limit  is  shown in an alarm color (default: blinking  red).  The

capture-to-disk will turn on when the first packet is copied, and

will turn off two minutes after the last.

If  you  touch the 'Escape' key you will bring up  the  following

Menu.

D Show Space on Disk

R Reset Packet Counters

Q Quit to Main Menu

Z Show Spacecraft data Files

Type the letter associated with the option to perform it.

Each of the options are described below.

8.1 Show Space on Disk

This  option  allows you to see how much space is left  on  a

disk with exiting from the program.

8.2 Reset Packet Counters

This  option  resets the packet counters to  zero.  use  this

before  a  pass  to see how many packets  of  each  type  are

received during the pass.

8.3 Quit to Main Menu

This option returns you to the Main menu.

8.4 Show Spacecraft data Files

This  option  shows  you  the  data  files  for  the   chosen

spacecraft in the default directory path.

9.0 (Data) Extraction Mode

In this mode, data is extracted from a playback file into a  file

that  can be read into a spreadsheet.  If you answer  the  prompt

for the default file with a non-existent filename, WHATS-UP  will

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 14.

prompt  you  for individual channel numbers.   To  terminate  the

sequence  and  begin  the extract mode,  touch  the  'Enter'  key

without entering a channel number.

Note: Start and stop times are text string matches.

10.0 Configuration File

The  contents  of the Configuration file  (WHATS-UP.SYS)  are  as

follows:

Your callsign (e.g. G3ZCZ)

Default configuration file) (e.g. Dove)

station latitude (e.g. 35.00)

station longitude (e.g. 74.00)

station altitude (e.g. 100)

default directory path (e.g C:)

default extracted data file (e.g. whats-up.txt)

default  file  name  with list  of  telemetry  parameters  to

extract file (e.g. ARRAYS)

TNC Type (e.g. PK-232)

PC serial port (e.g. 1)

PC serial baud rate (e.g. 1200)

data bits (e.g. 8)

Stop bits (e.g. 1)

parity (e.g. 0)

status (top) window color (e.g. 79)

Telemetry Page color (e.g. 14)

outgoing window color (e.g. 14)

incoming window color (e.g. 30)

prompt window color (e.g. 15)

alarm window color (e.g. 15)

bottom window color (e.g. 79)

Emphasis color (e.g. 14)

prompt color (e.g. 14)

option color (e.g. 78)

parameter changed color (e.g. 95)

parameter limit exceeded color (e.g. 14)

*  Comment line

remaining  lines  are  commands sent to the  PK232  when  you

configure the TNC. Note to avoid lock ups FLOW and XFLOW

MUST be OFF.

HEAD ON

ECHO OFF

DAYSTAMP ON

MONITOR 6

MSTAMP ON

FLOW OFF

XFLOW OFF

The callsign, TNC Type and geographic parameters are not used  in

this version.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 15.

You  must  configure  WHATS-UP  before  you  try  any  Real  Time

activity.

11.0 Spacecraft Parameter Files

You  will  need a Different spacecraft parameter  file  for  each

spacecraft.   Spacecraft  parameter  files  are  named   by   the

spacecraft   and  given  the  extension  '.SYS'.   Examples   are

'DOVE.SYS'  and  'Fuji20.SYS'.   These files  determine  how  the

individual channels are decoded, and where, in which screen page,

and  in which color the decoded data are displayed.  Some of  the

items  are  unique  to  WHATS-UP  and  some  to  the   particular

spacecraft.

The contents of the SPACECRAFT.SYS file are as described below.

11.1 Spacecraft ID.

This is the call sign of the spacecraft. For example,

Spacecraft      ID

DOVE-OSCAR 17  DOVE-1

Fuji-OSCAR 12  8J1JAS

Fuji-OSCAR 20  8J1JBS

In the case of the Microsats and Fuji-OSCAR 12/20,  WHATS-UP

searches the packet headers to detect the spacecraft by  the

ID.

11.2 Spacecraft Suffix

This becomes the filetype for the capture-to-disk files. The

default suggestions are as shown below.

Spacecraft       Suffix

AMSAT-OSCAR 13     O13

UoSAT-OSCAR 11     U11

DOVE-OSCAR 17      D17

Fuji-OSCAR 12      F12

Fuji-OSCAR 20      F20

PACSAT-OSCAR 16    P16

WEBER-OSCAR 18     W18

LU-OSCAR 19        L19

11.3 Selected or default display page number

This  is  the  default display page for  the  Real-time  and

Playback modes, when WHATS-UP is first loaded.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 16.

11.4 Number of display pages

This  is the number of display pages that you have  defined.

The maximum number is 16.  This number must be correct as it

tells WHATS-UP how many page definition lines to read.

11.5 Page Definitions

These are the page definitions, with two items on the  line.

The  format  is PAGE_TITLE, Page_Color, as  in  the  example

below.

SPACECRAFT HOUSEKEEPING, 30

11.6 Telemetry Parameter Configuration

The   next  set  of  items  are  the   Telemetry   parameter

configurations (maximum = 99). You must have at least one of

these  lines in the file. If you want a value to show up  in

more  than one page (other than the wild card [0]) you  must

enter it twice (once per page). Typically each row  contains

17 items in the format shown below.

TLM_Channel, TLM_Segment_ID, TLM_Description,  TLM_Eqn_Type,

TLM_Ceof_C,  TLM_Ceof_B,  TLM_Ceof_A,  TLM_Units,  TLM_Page,

TLM_Row,   TLM_Col,  TLM_Width,  TLM_Dec,   TLM_Limit_Check,

TLM_Limit_Low,TLM_Limit_High.

Each item is described in the following sections.

11.6.1 TLM_Channel

This is the channel number of the telemetry data in the

frame.  The  DOVE channel number is  hexadecimal  (e.g.

'0F'), Fuji is decimal.  Each entry must be two digits.

A special identifying TLM_Channel is defined in  WHATS-

UP. If the value is '99 then the segment identifier and

position  of the segment identifier is defined  in  tow

positions in the line.  This special channel identifies

the type of telemetry frame.

11.6.2 TLM_Segment_ID

This is the segment identifier.

The Fuji frame contains one real time segment  (Segment

1)  in a frame addressed as 8J1JBS>BEACON.   A  typical

frame is shown below.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 17.

19-Apr-90  17:14:34  8J1JBS*>BEACON:

JAS1b RA 90/04/19 17:13:58

609 430 687 676 744 837 845 829 498 681

617 001 505 516 526 524 526 523 654 000

683 675 686 695 999 643 875 471 099 000

110 111 000 000 111 100 001 111 111 000

The  segment  identifier is in the seventh  and  eighth

characters  of  the first line of the data.  A  segment

identifier  of  that  position  identifies  the  second

segment.  The segment identifier is the 'RA' located on

the  first line of the data just after the JAS1b  where

the 'R' in 'RA' is the seventh character in the line.

Any  telemetry  frame  addressed  to  BEACON   received

without that segment identifier is assumed by  WHATS-UP

to be Segment 2.

DOVE  transmits telemetry in two frames each  addressed

as  DOVE-1>TLM. The Microsat ASCII frame thus  contains

two segments.   Two typical segments of DOVE  telemetry

are shown below.

DOVE-1>TLM [01/29/90  22:08:46]:

00:59 01:59 02:86 03:30 04:58 05:58 06:6D 07:45 08:6C 09:66 0A:A1

0B:D9 0C:E8 0D:D8 0E:01 0F:23 10:CC 11:A8 12:00 13:01 14:A8 15:94

16:96 17:94 18:95 19:96 1A:93 1B:90 1C:9A 1D:98 1E:23 1F:5E 20:BC

DOVE-1>TLM [01/29/90  22:08:47]:

21:98 22:7B 23:24 24:21 25:2E 26:00 27:00 28:00 29:00 2A:00 2B:00

2C:00 2D:29 2E:00 2F:9B 30:C8 31:9C 32:11 33:DA 34:C0 35:95 36:A4

37:A4 38:B2 39:96 3A:00

The  default segment identifier used by WHATS-UP is  in

the  first and second characters of the first  line  of

the  data. A segment identifier of '00' identifies  the

first  segment,  and  anything else  in  that  position

identifies the second segment.

11.6.3 TLM_Description

This  item  is the text string or  description  of  the

telemetry channel that will be displayed on the  screen

page. (e.g. '+Z Array Temp.')

11.6.4 TLM_Eqn_Type

This item tells WHATS-UP the type of equation to use to

decode the telemetry.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 18.

A type 1 equation is a quadratic of the form

Y = A*N^2 + B*N + C,

where: N = raw telemetry data value

A, B, C = Equation Coefficients

Y = Result (In Specified Units)

This is the equation used by AMSAT-NA in the Microsats.

Fuji uses two other equations. They are in the  formats

of

Y = D*(N+E), and Y = F*(G-N).  If you know some algebra

you can convert both of Fuji's equations to the  Format

used  by AMSAT, but since a computer is  involved,  why

not let it do the job.  You do however have to  convert

an equation of the form Y=(N+a)/b.

A  type  2 equation in WHATS-UP has the format  in  the

form of

Y = B*(N-A) , C=0

where B, A are coefficients

Y,   N   are   decimal

values

A  type  3 equation in WHATS-UP has the format  in  the

form of

Y = B*(A-N) , C=0

where B, A are coefficients

Y, N  are decimal values

If the TLM_Channel is '99', then this item contains the

location of the segment identifier in the first line of

the telemetry data.

In the case of Fuji-OSCAR 12/20, the segment identifier

is the 'RA' located on the first line of the data  just

after  the  JAS1b  where the 'R' in  'RA'  is  the  7th

character  in  the first line of the  data.   Thus  the

position of the segment identifier in this case is 7.

In the case of the AMSAT Microsat ASCII telemetry,  the

segment identifier for the first segment is '00" in the

seventh character of the first line of the data.

Note that the segment identifier and position are  user

definable, and appear in the spacecraft.sys file.

11.6.5 TLM_Ceof_C

This item is the equation Coefficient C.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 19.

11.6.6 TLM_Ceof_B

This item is the equation Coefficient B.

11.6.7 TLM_Ceof_A

This item is the equation Coefficient A.

11.6.8 TLM_Units

This  item is the Units text string (e.g. '.C') in  the

screen  display.   However if the Tlm_Channel  is  '99'

then this item is the segment identifier string.

11.6.9 TLM_Page

This  item is the Display page number. A 0 is  a  'wild

card' which will be displayed on every page.

11.6.10 TLM_Row

This item is the Display page row.  It identifies which

row in the screen the data element will be displayed.

11.6.11 TLM_Col

This  item  is the Display page column.  It  identifies

which  column  in  the screen the  data  item  will  be

displayed.

11.6.12 TLM_Width

This  item is the Display width for Engineering  Units.

It tells WHATS-UP how many characters wide the  display

is  to be.  You can set it to any value you  want.  For

example,  you  can  display  a  voltage  as  '1.3'   or

'1.28567'.   Before  you widen the  display  too  much,

remember the sampling accuracy of the analog-to-digital

converter in the spacecraft.

11.6.13 TLM_Dec

This  item  defines  the number  of  digits  after  the

decimal point in the display.

11.6.14 TLM_Limit_Check

This  item tells WHATS-UP to perform limit checking  on

the  telemetry channel. It may have several  values  as

described below.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 20.

0 = do nothing,

1 = check below low limit,

2= check above high limit,

3 = check for [below low limit] or [above high limit]).

11.6.15 TLM_Limit_Low

This item is the Low limit value (e.g. -4.00).

11.6.17 TLM_Limit_High

This item is the  High limit value (e.g. +10.6).

A  line with an '*' as the first character  terminates  this

section.

11.7 Packet/Link Parameters

The  next  set  of  items  are  the  Packet/Link  Parameters

configurations  (maximum =*** ). You must have at least  one

of  these lines in the file. If you want a value to show  up

in  more  than one page (other than the wild card  [0])  you

must  enter  it twice (once per page).  Typically  each  row

contains 10 items in the format shown below.

Packet_title,   Packet_Type,   Packet_Lines,    Packet_Page,

Packet_Color,  Packet_Row, Packet_Col, Link_Page,  Link_Row,

Link_Col.

Each item is described in the following sections.

11.7.1 Packet_title

This  item is the name of UNP address  (e.g.  TLM,WASH,

BCXRT).

11.7.2 Packet_Type

This  item  is used by WHATS-UP to define the  type  of

packet.  The following assignments may be used.

1    AMSAT  Microsat with the format CC:DD where CC  is

the   hexadecimal  channel  number  and   DD   the

hexadecimal data.

3    Fuji format of decimal data in which the line  and

the  position  on the line identify  the  channel.

WHATS-UP allows for up to 60 channels.

11.7.3 Packet_Lines

This item is the number of lines of text in the packet.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 21.

for  example, the AMSAT TLM packets contain  three

lines, the WASH packets contain only one.

11.7.4 Packet_Page

This item  is  the page that the raw  contents  of  the

packet will be displayed on. A '0' is a wild  card

which will make WHATS-UP display it on every page.

By careful use of this item, you can display  both

raw and decoded packet data on the same page.

11.7.5 Packet_Color

This item is the color that the raw packet data will be

displayed in.

11.7.6 Packet_Row

This item is the row position that the raw packet  will

be displayed in, on the selected page.

11.7.7 Packet_Col

This item  is the column position that the  raw  packet

will be displayed in, on the selected page.

11.7.8 Link_Page

This item is the Display page for the cumulative  count

of the packet type. The wild card '0' applies.

11.7.9 Link_Row

This item  is the row position that the  packet  header

will be displayed in, on the selected page.

11.7.10 Link_Col

This item is the column position that the packet header

will be displayed in, on the selected page.

These lines also terminate with an '*' character.

12.0 Telemetry Channel Extraction File

The  contents of this file are the defaults for  extracting  data

from the playback file.  A typical set are shown below.  WHATS-UP

does  a  string  match,  and looks for  the  first  time  that  a

particular string occurs. You may thus use the contents of a time

packet, or the time mark in a header.

ZCZC (default start time string) {start of file}

NNNN (default stop time string) {end of file}

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP.DOC Release 0.55 Page 22.

26 (list of telemetry channels to be extracted)

27 (This one contains the solar array currents)

28

29

2A

2B

13.0 Extracted Telemetry Data File

This an ASCII string, comma delimited file which can be  imported

into your spreadsheet.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX A AMSAT Microsat ASCII Telemetry  Page 23

Latest Microsat Telemetry (TLM) Equations

By  Jan  King W3GEY  Reprinted from the AMSAT Journal  Volume  13

Number 1, March 1990.

Spacecraft:  PACSAT-1:                            Rev: 1

Date: 1/7/90

Equations are in the form: Y = A*N^2 + B*N + C

where:

N = Telemetry Count (00 - FF)

A, B, C = Equation Coefficients

Y = Result (In Specified Units)

HEX   Description:        C:           B:           A:      Units:

cccccccccc   bbbbbbbbbb   aaaaaaaaaa  uuuuuu

0    Rx D DISC:     +9.202       -0.08990     0.000       kHz

1    Rx D S meter:  +0.000       +1.000       0.000       Counts

2    Rx C DISC:     +9.179       -0.09277     0.000       kHz

3    Rx C S meter:  +0.000       +1.000       0.000       Counts

4    Rx B DISC:     +9.837       -0.08838     0.000       kHz

5    Rx B S meter:  +0.000       +1.000       0.000       Counts

6    Rx A DISC:     +9.779       -0.09144     0.000       kHz

7    Rx A S meter:  +0.000       +1.000       0.000       Counts

8    Rx E/F DISC:   +10.817      -0.09911     0.000       kHz

9    Rx E/F S meter:+0.000       +1.000       0.000       Counts

A    +5 Volt Bus:   +0.000       +0.0305      0.000       Volts

B    +5V Rx Current:+0.000       +0.000250    0.000       Amps

C    +2.5V  VREF:   +0.000       +0.0108      0.000       Volts

D    8.5V BUS:      +0.000       +0.0391      0.000       Volts

E    IR Detector:   +0.000       +1.000       0.000       Counts

F    LO Monitor I:  +0.000       +0.000037    0.000       Amps

10    +10V Bus:      +0.000       +0.0500      0.000       Volts

11    GASFET Bias I: +0.000       +0.000026    0.000       Amps

12    Ground REF:    +0.000       +0.0100      0.000       Volts

13    +Z Array V:    +0.000       +0.1023      0.000       Volts

14    Rx Temp:       +101.05      -0.6051      0.000       Deg. C

15    +X (RX) temp:  +101.05      -0.6051      0.000       Deg. C

16    Bat 1 V:       +1.8225      -0.0038046   0.000       Volts

17    Bat 2 V:       +1.9418      -0.0046890   0.000       Volts

18    Bat 3 V:       +1.8699      -0.0041641   0.000       Volts

19    Bat 4 V:       +1.7403      -0.0032880   0.000       Volts

1A    Bat 5 V:       +1.8792      -0.0042492   0.000       Volts

1B    Bat 6 V:       +2.0499      -0.0054532   0.000       Volts

1C    Bat 7 V:       +1.9062      -0.0045331   0.000       Volts

1D    Bat 8 V:       +1.7536      -0.0033192   0.000       Volts

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX A AMSAT Microsat ASCII Telemetry  Page 24

1E    Array V:       +8.055       +0.06790     0.000       Volts

1F    +5V Bus:       +2.035       +0.0312      0.000       Volts

20    +8.5V Bus:     +5.464       +0.0184      0.000       Volts

21    +10V Bus:      +7.650       +0.0250      0.000       Volts

22    BCR Set Point: -6.1130      +1.1270      0.000       Counts

23    BCR Load Cur:  -0.0477      +0.00767     0.000       Amps

24    +8.5V Bus Cur: -0.00179     +0.000894    0.000       Amps

25    +5V Bus Cur:   -0.00104     +0.00406     0.000       Amps

26    -X Array Cur:  -0.00995     +0.00243     0.000       Amps

27    +X Array Cur:  -0.02370     +0.00254     0.000       Amps

28    -Y Array Cur:  -0.02220     +0.00273     0.000       Amps

29    +Y Array Cur:  -0.01810     +0.00259     0.000       Amps

2A    -Z Array Cur:  -0.02230     +0.00221     0.000       Amps

2B    +Z Array Cur:  -0.02000     +0.00232     0.000       Amps

2C    Ext Power Cur: -0.02000     +0.00250     0.000       Amps

2D    BCR Input Cur: -0.02345     +0.00355     0.000       Amps

2E    BCR Output Cur:+0.00869     +0.00303     0.000       Amps

2F    Bat 1 Temp:    +101.05      -0.6051      0.000       Deg. C

30    Bat 2 Temp:    +101.05      -0.6051      0.000       Deg. C

31    Baseplt Temp:  +101.05      -0.6051      0.000       Deg. C

32    PSK TX RF Out: -0.0291      +0.00361     +0.0000869  Watts

33    RC PSK TX Out: +0.0055      +0.00172     +0.0001180  Watts

34    PSK TX HPA Temp+101.05      -0.6051      0.000       Deg. C

35    +Y Array Temp: +101.05      -0.6051      0.000       Deg. C

36    RC PSK HPA Temp+101.05      -0.6051      0.000       Deg. C

37    RC PSK BP Temp:+101.05      -0.6051      0.000       Deg. C

38    +Z Array Temp: +101.05      -0.6051      0.000       Deg. C

39    S band TX Out: -0.0088      +0.00435     0.000       Watts

3A    S band HPA Temp 0.000       +1.000       0.000       Counts

ADC Equations: V = 0.01028 N - 0.02055

N = 97.31 V +2.000

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX A AMSAT Microsat ASCII Telemetry  Page 25

Spacecraft:  DOVE-1:                              Rev: 1

Date: 1/7/90

Equations are in the form: Y = A*N^2 + B*N + C

where:

N = Telemetry Count (00 - FF)

A, B, C = Equation Coefficients

Y = Result (In Specified Units)

HEX   Description:        C:           B:           A:      Units:

cccccccccc   bbbbbbbbbb   aaaaaaaaaa  uuuuuu

0    Rx E/F Audio(W)+0.000       +0.0246      0.000       V(p-p)

1    Rx E/F Audio(N)+0.000       +0.0246      0.000       V(p-p)

2    Mixer Bias V:  +0.000       +0.0102      0.000       Volts

3    Osc. Bisd V:   +0.000       +0.0102      0.000       Volts

4    Rx A Audio (W):+0.000       +0.0246      0.000       V(p-p)

5    Rx A Audio (N):+0.000       +0.0246      0.000       V(p-p)

6    Rx A DISC:     +10.427      -0.09274     0.000       kHz

7    Rx A S meter:  +0.000       +1.000       0.000       Counts

8    Rx E/F DISC:   +9.6234      -0.09911     0.000       kHz

9    Rx E/F S meter:+0.000       +1.000       0.000       Counts

A    +5 Volt Bus:   +0.000       +0.0305      0.000       Volts

B    +5V Rx Current:+0.000       +0.000100    0.000       Amps

C    +2.5V  VREF:   +0.000       +0.0108      0.000       Volts

D    8.5V BUS:      +0.000       +0.0391      0.000       Volts

E    IR Detector:   +0.000       +1.000       0.000       Counts

F    LO Monitor I:  +0.000       +0.000037    0.000       Amps

10    +10V Bus:      +0.000       +0.05075     0.000       Volts

11    GASFET Bias I: +0.000       +0.000026    0.000       Amps

12    Ground REF:    +0.000       +0.0100      0.000       Volts

13    +Z Array V:    +0.000       +0.1023      0.000       Volts

14    Rx Temp:       +101.05      -0.6051      0.000       Deg. C

15    +X (RX) temp:  +101.05      -0.6051      0.000       Deg. C

16    Bat 1 V:       +1.7932      -0.0034084   0.000       Volts

17    Bat 2 V:       +1.7978      -0.0035316   0.000       Volts

18    Bat 3 V:       +1.8046      -0.0035723   0.000       Volts

19    Bat 4 V:       +1.7782      -0.0034590   0.000       Volts

1A    Bat 5 V:       +1.8410      -0.0038355   0.000       Volts

1B    Bat 6 V:       +1.8381      -0.0038450   0.000       Volts

1C    Bat 7 V:       +1.8568      -0.0037757   0.000       Volts

1D    Bat 8 V:       +1.7868      -0.0034068   0.000       Volts

1E    Array V:       +7.205       +0.07200     0.000       Volts

1F    +5V Bus:       +1.932       +0.0312      0.000       Volts

20    +8.5V Bus:     +5.265       +0.0173      0.000       Volts

21    +10V Bus:      +7.469       +0.021765    0.000       Volts

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX A AMSAT Microsat ASCII Telemetry  Page 26

22    BCR Set Point: -8.762       +1.1590      0.000       Counts

23    BCR Load Cur:  -0.0871      +0.00698     0.000       Amps

24    +8.5V Bus Cur: -0.00920     +0.001899    0.000       Amps

25    +5V Bus Cur:   +0.00502     +0.00431     0.000       Amps

26    -X Array Cur:  -0.01075     +0.00215     0.000       Amps

27    +X Array Cur:  -0.01349     +0.00270     0.000       Amps

28    -Y Array Cur:  -0.01196     +0.00239     0.000       Amps

29    +Y Array Cur:  -0.01141     +0.00228     0.000       Amps

2A    -Z Array Cur:  -0.01653     +0.00245     0.000       Amps

2B    +Z Array Cur:  -0.01137     +0.00228     0.000       Amps

2C    Ext Power Cur: -0.02000     +0.00250     0.000       Amps

2D    BCR Input Cur: +0.06122     +0.00317     0.000       Amps

2E    BCR Output Cur:-0.01724     +0.00345     0.000       Amps

2F    Bat 1 Temp:    +101.05      -0.6051      0.000       Deg. C

30    Bat 2 Temp:    +101.05      -0.6051      0.000       Deg. C

31    Baseplt Temp:  +101.05      -0.6051      0.000       Deg. C

32    FM TX#1 RF OUT:+0.0256      -0.000884    +0.0000836  Watts

33    FM TX#2 RF OUT:-0.0027      +0.001257    +0.0000730  Watts

34    PSK TX HPA Temp+101.05      -0.6051      0.000       Deg. C

35    +Y Array Temp: +101.05      -0.6051      0.000       Deg. C

36    RC PSK HPA Temp+101.05      -0.6051      0.000       Deg. C

37    RC PSK BP Temp:+101.05      -0.6051      0.000       Deg. C

38    +Z Array Temp: +101.05      -0.6051      0.000       Deg. C

39    S band TX Out: -0.0451      +0.00403     0.000       Watts

3A    S band HPA Temp+101.05      -0.6051      0.000       Deg. C

ADC Equations: V = 0.01028 N - 0.05138

N = 97.31 V +5.000

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX A AMSAT Microsat ASCII Telemetry  Page 27

Spacecraft:  WEBER-1:                            Rev: 1

Date: 1/7/90

Equations are in the form: Y = A*N^2 + B*N + C

where:

N = Telemetry Count (00 - FF)

A, B, C = Equation Coefficients

Y = Result (In Specified Units)

HEX   Description:        C:           B:           A:      Units:

cccccccccc   bbbbbbbbbb   aaaaaaaaaa  uuuuuu

0    Rx D DISC:     +11.087      -0.08949     0.000       kHz

1    Rx D S meter:  +0.000       +1.000       0.000       Counts

2    Rx C DISC:     +10.322      -0.09448     0.000       kHz

3    Rx C S meter:  +0.000       +1.000       0.000       Counts

4    Rx B DISC:     +10.348      -0.09004     0.000       kHz

5    Rx B S meter:  +0.000       +1.000       0.000       Counts

6    Rx A DISC:     +11.387      -0.09535     0.000       kHz

7    Rx A S meter:  +0.000       +1.000       0.000       Counts

8    Rx E/F DISC:   +10.746      -0.09348     0.000       kHz

9    Rx E/F S meter:+0.000       +1.000       0.000       Counts

A    +5 Volt Bus:   +0.000       +0.03523     0.000       Volts

B    +5V Rx Current:+0.000       +0.000234    0.000       Amps

C    +2.5V  VREF:   +0.000       +0.0133      0.000       Volts

D    8.5V BUS:      +0.000       +0.0524      0.000       Volts

E    IR Detector:   +0.000       +1.000       0.000       Counts

F    LO Monitor I:  +0.000       +0.000033    0.000       Amps

10    +10V Bus:      +0.000       +0.0767      0.000       Volts

11    GASFET Bias I: +0.000       +0.000026    0.000       Amps

12    Ground REF:    +0.000       +0.0100      0.000       Volts

13    +Z Array V:    +0.000       +0.1023      0.000       Volts

14    Rx Temp:       +100.01      -0.5980      0.000       Deg. C

15    +X (RX) Temp:  +100.01      -0.5980      0.000       Deg. C

16    Bat 1 V:       +1.8292      -0.0037196   0.000       Volts

17    Bat 2 V:       +1.8202      -0.0036943   0.000       Volts

18    Bat 3 V:       +1.8050      -0.0036721   0.000       Volts

19    Bat 4 V:       +1.8576      -0.0038979   0.000       Volts

1A    Bat 5 V:       +1.8095      -0.0037439   0.000       Volts

1B    Bat 6 V:       +1.8979      -0.0041754   0.000       Volts

1C    Bat 7 V:       +1.8246      -0.0038126   0.000       Volts

1D    Bat 8 V:       +1.7486      -0.0030475   0.000       Volts

1E    Array V:       +7.800       +0.06790     0.000       Volts

1F    +5V Bus:       +1.838       +0.0312      0.000       Volts

20    +8.5V Bus:     +5.793       +0.0184      0.000       Volts

21    +10V Bus:      +7.650       +0.0250      0.000       Volts

22    BCR Set Point: -6.1963      +1.1277      0.000       Counts

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX A AMSAT Microsat ASCII Telemetry  Page 28

23    BCR Load Cur:  -0.0405      +0.00620     0.000       Amps

24    +8.5V Bus Cur: +0.00384     +0.000830    0.000       Amps

25    +5V Bus Cur:   -0.00763     +0.00394     0.000       Amps

26    -X Array Cur:  -0.00140     +0.00210     0.000       Amps

27    +X Array Cur:  +0.00946     +0.00226     0.000       Amps

28    -Y Array Cur:  -0.01018     +0.00224     0.000       Amps

29    +Y Array Cur:  -0.01168     +0.00239     0.000       Amps

2A    -Z Array Cur:  -0.01516     +0.00237     0.000       Amps

2B    +Z Array Cur:  -0.02111     +0.00239     0.000       Amps

2C    Ext Power Cur: -0.02000     +0.00250     0.000       Amps

2D    BCR Input Cur: -0.02189     +0.00332     0.000       Amps

2E    BCR Output Cur:-0.03019     +0.00327     0.000       Amps

2F    Bat 1 Temp:    +100.01      -0.5980      0.000       Deg. C

30    Bat 2 Temp:    +100.01      -0.5980      0.000       Deg. C

31    Baseplate Temp:+100.01      -0.5980      0.000       Deg. C

32    PSK TX RF Out: +0.2104      -0.01203     +0.0001786  Watts

33    RC PSK TX Out: +0.0340      -0.00969     +0.0002198  Watts

34    PSK TX HPA Temp+100.01      -0.5980      0.000       Deg. C

35    +Y Array Temp: +100.01      -0.5980      0.000       Deg. C

36    RC PSK HPA Temp+100.01      -0.5980      0.000       Deg. C

37    RC PSK BP Temp:+100.01      -0.5980      0.000       Deg. C

38    +Z Array Temp: +0.0000      +1.0000      0.000       Counts

ADC Equations: V = 0.01016 N - 0.05080

N = 98.43 V +5.000

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX A AMSAT Microsat ASCII Telemetry  Page 29

Spacecraft:  LUSAT-1:                             Rev: 1

Date: 1/7/90

Equations are in the form: Y = A*N^2 + B*N + C

where:

N = Telemetry Count (00 - FF)

A, B, C = Equation Coefficients

Y = Result (In Specified Units)

HEX   Description:        C:           B:           A:      Units:

cccccccccc   bbbbbbbbbb   aaaaaaaaaa  uuuuuu

0    Rx D DISC:     +9.802       -0.08779     0.000       kHz

1    Rx D S meter:  +0.000       +1.000       0.000       Counts

2    Rx C DISC:     +8.429       -0.09102     0.000       kHz

3    Rx C S meter:  +0.000       +1.000       0.000       Counts

4    Rx B DISC:     +9.291       -0.08317     0.000       kHz

5    Rx B S meter:  +0.000       +1.000       0.000       Counts

6    Rx A DISC:     +9.752       -0.08310     0.000       kHz

7    Rx A S meter:  +0.000       +1.000       0.000       Counts

8    Rx E/F DISC:   +10.110      -0.08610     0.000       kHz

9    Rx E/F S meter:+0.000       +1.000       0.000       Counts

A    +5 Volt Bus:   +0.000       +0.0305      0.000       Volts

B    +5V Rx Current:+0.000       +0.000250    0.000       Amps

C    +2.5V  VREF:   +0.000       +0.0108      0.000       Volts

D    8.5V BUS:      +0.000       +0.0391      0.000       Volts

E    IR Detector:   +0.000       +1.000       0.000       Counts

F    LO Monitor I:  +0.000       +0.000037    0.000       Amps

10    +10V Bus:      +0.000       +0.0508      0.000       Volts

11    GASFET Bias I: +0.000       +0.000026    0.000       Amps

12    Ground REF:    +0.000       +0.0100      0.000       Volts

13    +Z Array V:    +0.000       +0.1023      0.000       Volts

14    Rx Temp:       +93.24       -0.5609      0.000       Deg. C

15    +X (RX) Temp:  +93.24       -0.5609      0.000       Deg. C

16    Bat 1 V:       +1.7343      -0.0029740   0.000       Volts

17    Bat 2 V:       +1.7512      -0.0032113   0.000       Volts

18    Bat 3 V:       +1.7790      -0.0034038   0.000       Volts

19    Bat 4 V:       +1.7286      -0.0030036   0.000       Volts

1A    Bat 5 V:       +1.8114      -0.0036960   0.000       Volts

1B    Bat 6 V:       +1.7547      -0.0032712   0.000       Volts

1C    Bat 7 V:       +1.7151      -0.0030739   0.000       Volts

1D    Bat 8 V:       +1.6846      -0.0028534   0.000       Volts

1E    Array V:       +8.100       +0.06790     0.000       Volts

1F    +5V Bus:       +2.035       +0.0312      0.000       Volts

20    +8.5V Bus:     +5.614       +0.0184      0.000       Volts

21    +10V Bus:      +7.650       +0.0250      0.000       Volts

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX A AMSAT Microsat ASCII Telemetry  Page 30

22    BCR Set Point: +3.7928      +1.0616      0.000       Counts

23    BCR Load Cur:  -0.0244      +0.00628     0.000       Amps

24    +8.5V Bus Cur: +0.00412     +0.000773    0.000       Amps

25    +5V Bus Cur:   +0.02461     +0.00438     0.000       Amps

26    +X Array Cur:  -0.01614     +0.00232     0.000       Amps

27    -X Array Cur:  -0.01158     +0.00238     0.000       Amps

28    -Y Array Cur:  +0.00278     +0.00206     0.000       Amps

29    +Y Array Cur:  +0.00136     +0.00218     0.000       Amps

2A    -Z Array Cur:  +0.00370     +0.00209     0.000       Amps

2B    +Z Array Cur:  -0.00793     +0.00216     0.000       Amps

2C    Ext Power Cur: -0.02000     +0.00250     0.000       Amps

2D    BCR Input Cur: -0.00901     +0.00283     0.000       Amps

2E    BCR Output Cur:+0.00663     +0.00344     0.000       Amps

2F    Bat 1 Temp:    +93.24       -0.5609      0.000       Deg. C

30    Bat 2 Temp:    +93.24       -0.5609      0.000       Deg. C

31    Baseplt Temp:  +93.24       -0.5609      0.000       Deg. C

32    PSK TX RF Out: +0.1059      +0.00095     +0.0000834  Watts

33    RC PSK TX Out: +0.0178      +0.00135     +0.0000833  Watts

34    PSK TX HPA Temp+93.24       -0.5609      0.000       Deg. C

35    +Y Array Temp: +93.24       -0.5609      0.000       Deg. C

36    RC PSK HPA Temp+93.24       -0.5609      0.000       Deg. C

37    RC PSK BP Temp:+93.24       -0.5609      0.000       Deg. C

38    +Z Array Temp: +93.24       -0.5609      0.000       Deg. C

39    LU Bcn Temp A: +93.24       -0.5609      0.000    *  Deg. C

3A    LU Bcn Temp D: +93.24       -0.5609      0.000    ** Deg. C

3B    Coax Rly Stat: +0.000       +1.0000      0.000       Counts

3C    Coax Rly Stat: +0.000       +1.0000      0.000       Counts

ADC Equations: V = 0.00953 N

N = 104.94 V

* Note 1:  Thermistor located near box center adjacent to LU

thermistor channel no. 5.

**  Note  2:   Thermistor  located  near -X face of  box  on  the

experiment baseplate.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX B The Fuji-OSCAR 20 Spacecraft Page 31

Introduction

On  February  7 1990, the National Space  Development  Agency  of

Japan (NASDA) put the Marine Observation Satellite (MOS) 1b  into

orbit.   The launch vehicle also carried two secondary  payloads,

Fuji-Oscar  20 and the Deployable Boom and Umbrella Test  (DEBUT)

spacecraft which is similar in shape and weight to Fuji-OSCAR 20.

MOS-1b  was  placed into a circular polar orbit, then  DEBUT  and

Fuji-OSCAR  20 separated from the launch vehicle at  0233,  above

Santiago, Chile. First signals from the spacecraft were  received

in Tokyo around 0309 UTC.

Fuji-OSCAR  20 is similar in construction to Fuji-OSCAR  12.   In

fact, it was originally constructed as a backup to Fuji-OSCAR  12

and designated as JAS-1B. It has since been modified and improved

as  a  result of the lessons learned during the flight  of  Fuji-

OSCAR  12.  Fuji-OSCAR 12 was known as Fuji-1 in Japan,  so  this

spacecraft  is known by the Japanese as Fuji-2 and as  Fuji-OSCAR

12 (or FO-12) by the rest of the world.  This article,  describes

the spacecraft and its mission.

The Orbit

Fuji-OSCAR  20's  planned service life is 5 years.  It  is  in  a

sunsynchronous elliptical polar orbit, having a perigee of  about

900  km  and  an apogee of about 1740 at  an  inclination  of  99

degrees.  The  Period of the orbit is about  105  minutes.   This

orbit  is  optimal  for MOS-1b which is  to  study  oceanographic

resources and observe agricultural environmental conditions.   In

this  orbit, the spacecraft passes over a given line of  latitude

at  approximately  the same time each day.  In  this  orbit,  the

spacecraft is shielded from the sun by the earth for about 33% of

the  time.  This  eclipse means that the  solar  cells  can  only

provide power for about 70 minutes in each orbit and that the on-

board  nickel  cadmium  storage  batteries  have  to  power   the

spacecraft for the remaining 35 minutes.

The Spacecraft

Fuji-OSCAR  20  weighs about 50 kg. and is  a  polyhedron  shaped

spacecraft  440mm  in  diameter and 470mm in  height  covered  by

approximately  1500  gallium arsenide solar cells  which  provide

about  11  Watts of power to keep the 11  series-connected  NiCad

cells  (rectangular) with a capacity of 6 AH charged.  There  are

26  sides to the polyhedron which almost makes it  spherical  for

all  practical  purposes other than sticking solar cells  to  it.

Fuji-OSCAR  12  was  the same shape but only  carried  about  600

cells.  This larger number of cells means that Fuji-OSCAR 20  has

a positive power budget and should not need to be switched off to

recharge.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX B The Fuji-OSCAR 20 Spacecraft Page 32

The  Power  supply converts the raw bus voltage of +11 to  +18  V

(+14 V average) to the three regulated voltages (+10 V, +5 V,  -5

V)  used by the rest of the satellite with an efficiency  greater

than 70%.

The  attitude of the satellite is maintained by using the  torque

generated  by the interaction of two permanent magnets  with  the

earth's  magnetic field. This is a fairly conventional  technique

used  in the OSCAR series. Temperature stability is  achieved  by

using thermal insulation.

The Payload.

Fuji-OSCAR 20 carries two Mode J transponders, both of which  may

be operational at the same time.  One transponder is analog (Mode

JA), the other is digital (Mode JD).

The  frequencies and capabilities of the analog  transponder  are

similar  to those of Fuji-OSCAR 12.  It consists of  an  inverted

hetrodyne  linear  translator  with  a  passband  100  kHz  wide,

operating  with a mode J Uplink passband of 145.9 to 146.00  MHz,

and a corresponding Downlink Passband of 435.9 to 435.8 MHz.  The

spacecraft  has a Transmitter Output of approximately 1 watt.   A

ground station needs an Uplink EIRP of about 100 W to communicate

through  the  transponder.  The JA telemetry  beacon  is  on  the

nominal frequency of 435.795 MHz with a power output of about 100

mW and can use CW or PSK modulation.  Fuji-OSCAR 20 is using  the

callsign  8J1JBS and the beacons transmit telemetry in  the  same

manner as Fuji-OSCAR 12.

The   digital  transponder  provides   store-and-forward   packet

communication  using  the AX.25 link level protocol,  version  2.

Stations  who  used Fuji-OSCAR 12 are able to use  Fuji-OSCAR  20

without making any modifications to their equipment.  The  uplink

requires Bi-phased Manchester code on an FM signal, at a bit rate

of  1200 bps. There are 4 Uplink Frequencies: 145.85 MHz,  145.87

MHz, 145.89 MHz, 145.91 MHz.  The necessary ground station Uplink

EIRP is also about 100 W.  The transponder has an output power of

about 1 W on a downlink frequency of 435.91 MHz and uses NRZI PSK

at  1200 bps.  The same PSK modem used to copy Fuji-OSCAR  12  or

the  Microsats  is needed to copy Fuji-OSCAR  20.   The  downlink

channel also carries packet telemetry.

The 144 MHz receiving antenna is a ring turnstile mounted at  the

bottom of the side panels. The 435 MHz transmitting antenna is  a

turnstile antenna mounted at the top of satellite.  Both antennas

are  circularly  polarized.   Ground tests have  shown  that  the

transmitting  antenna is more omnidirectional than that of  Fuji-

OSCAR  12, however due to the structure of the  hybrid  circuitry

which  allow  both transponders to share the  same  antenna,  the

sense  of the circular polarization on the downlink is  different

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX B The Fuji-OSCAR 20 Spacecraft Page 33

for  each  mode.   As  the  apparent  polarization  is  different

depending  on  the  geometry  between  the  spacecraft  and   the

groundstation, you will probably have to change between left hand

and   right  hand  circular  polarization  during  a  pass.   The

spacecraft  is designed so that you can usually keep  the  uplink

and downlink polarization the same.

Mode  JA has provided strong trans-Atlantic signals and  many  CW

and  SSB QSOs. Mode-JD was switched on for the first time  during

Orbit #95.  To Digipeat via Fuji-OSCAR 20 you don't need to use a

digipeater  call.  With the present version of the software,  all

AX.25  frames  with a valid CRC heard by the spacecraft  will  be

digipeated.

The first claimed QSO on mode JD is by DB0OS when he connected to

himself  and  an extract from the information he copied  at  that

time is shown in Table 2.

----------------------------------------------------------------

Table  2  First European MODE-JD Self-Contact via  FUJI  OSCAR-20

made!

Telemetry and Self-Connect:

--------------------------

fm 8J1JBS to BEACON ctl UI^ pid F0

JAS1b RA 90/02/14 11:23:30

551 427 695 699 741 837 841 821 474 638

617 001 507 517 531 527 533 532 654 000

681 665 661 686 999 643 874 438 046 000

110 111 000 000 100 000 001 111 111 000

fm DB2OS to DB2OS ctl RR1-

fm DB2OS to DB2OS ctl I11^ pid F0

DB2OS de DB2OS (14.2.1990 um 11:15 utc)

fm DB2OS to DB2OS ctl I12^ pid F0

1st QSO via FUJI OSCAR-20

fm DB2OS to DB2OS ctl RR3v

fm 8J1JBS to BEACON ctl UI^ pid F0

JAS1b M0 90/02/14 11:26:00

Repeater is at your service from90/02/12 03:05:00

The JD Transmitter is available in all orbits

during JD mode.

----------------------------------------------------

The  spacecraft also carries a BBS which is accessed by means  of

the same commands used to access a terrestrial WA7MBL/W0RLI/AA4RE

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX B The Fuji-OSCAR 20 Spacecraft Page 34

type  of BBS.  You access the BBS by connecting to 8J1JBS on  any

of the four uplink channels. When you do connect to it, make sure

that you disconnect before LOS because Fuji-OSCAR 20 only  allows

16  simultaneous connections. Stations that hang in  there  after

the  satellite  drops below their local horizon block  access  by

other  stations  and have been christened  'Zombies'.    The  BBS

program  is  a modified version of the BBS  program  written  for

Fuji-OSCAR 12 and allows the use of 4 banks (1Mbyte) of memory. A

typical list of messages copied by KI6QE is shown in figure 2.

---------------------------------------------------

Figure 2 Typical Message Listing from the BBS (copied by KI6QE)

Fuji-OSCAR 20/JAS1b Mailbox ver. 2.00

commands [B/F/H/M/R/U/W]

Use H command for Help

JAS>JAS>NO.  DATE   UTC    FROM    TO      SUBJECT

0086 04/13 05:15  WB6GFJ  W6SHP   Welcome

0085 04/13 05:14  WB6LLO  KI6QE   SOFTWARE

0084 04/13 05:14  WB6GFJ  W9FMW   Our Chat

0082 04/13 03:38  W9FMW   WA4EJR  MESSAGE ON CIS

0080 04/13 03:36  KG6EX   N1GCR   From Ashley

0078 04/13 03:32  KG6EX   KD8SI   From Ashley

0077 04/13 03:31  KG6EX   N8AM    From Ashley

0076 04/13 03:30  KG6EX   DD4YR   From Ashley

0075 04/13 03:27  KG6EX   DL1CR   From Ashley

0074 04/13 03:25  KG6EX   G3RUH   From Ashley

---------------------------------------------------------

The Telemetry

The  spacecraft telemetry is transmitted either as CW or as  PSK.

The  CW  telemetry monitors 12 analog data points and  33  status

points,  the PSK telemetry monitors 29 analog data points and  33

status  points.  A typical set of PSK telemetry packets  captured

by  KI6QE is shown in figure 3. The telemetry decoding  equations

are  shown in Table 3.  As you can see the format of the data  in

the packet and the type of equations used are different to  those

used on the Microsats.

--------------------------------------------------------------

Figure 3 Fuji-OSCAR 20 PSK telemetry (as copied by KI6QE)

03-Apr-90  17:40:32  8J1JBS*>BEACON:

JAS1b RA 90/04/03 17:45:18

554 433 700 686 757 837 841 823 398 666

617 001 503 516 526 523 526 523 654 000

683 675 685 684 999 643 875 316 002 000

110 111 000 000 100 000 001 011 111 000

03-Apr-90  17:40:34  8J1JBS*>BEACON:

JAS1b RA 90/04/03 17:45:20

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX B The Fuji-OSCAR 20 Spacecraft Page 35

566 427 699 705 746 837 841 824 541 659

617 001 503 516 526 523 526 523 654 000

683 675 686 683 999 642 874 316 002 000

110 111 000 000 100 000 001 011 111 000

The Telemetry Formats of JAS-1b/Fuji-OSCAR 20

Telemetry data from FO-20 is transmitted on both the mode JA  and

JD beacons. Mode JA sends data by Morse code on the beacon signal

of  435.795  MHz, repeating one frame every one minute.  Mode  JD

sends a telemetry packet every 2 seconds on the digital  downlink

channel  of  435.91  MHz when the telemetry  mode  is  operating,

otherwise, one frame is downlinked every one minute. This article

contains  the information you need to decode the telemetry.   The

spacecraft  can downlink up to 30 items of data and 31  items  of

status in the telemetry.  The Mode JA beacon however only carries

12 data elements and most of status bytes.

Mode JA Telemetry Data

The  Mode  JA beacon transmits the telemetry data in  the  format

shown below. These data are sent by Morse code with a "HI HI"  at

the  beginning  of  each  frame,  with  a  speed  of  about   100

characters every minute,  and always in this format repeatedly.

HI  HI  1A  1B  1C  1D

2A  2B  2C  2D

3A  3B  3C  3D

4A  4B  4C  4D

5A  5B  5C  5D

How to Decode Mode JA telemetry

The  number  identifies the group, the letters A  through  D  are

decimal values expressed in two digits. Let this two-digit be  N,

for  each item, true value or engineering  value is  obtained  by

decoding  N as shown below.  For example, a value of 123  for  1A

means   group 1 and 23 is the measured value of the  solar  array

current.   Groups  4 and 5 contain status information  about  the

bird,   where  A,  B,  C  and  D  represent    octal    two-digit

combinations of 00 through 37. This corresponds to a  combination

of five binary  digits. Each bit shows status of each  designated

item  in the order from MSB (Most Significant Bit) to LSB  (Least

Significant Bit).

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX B The Fuji-OSCAR 20 Spacecraft Page 36

Mode JA Telemetry Conversion Equations

=========================================================

CH  DESCRIPTION                       CALIBRATION

=========================================================

1A  total solar array current          19x(N+0.4)   mA

1B  battery charge/discharge current  -38x(N-50)    mA

1C  battery voltage                   (N+4)x0.22    V

1D  center tap voltage of battery     (N+4)x0.1     V

2A  bus voltage                       (N+4)x0.2     V

2B  +5 V regulator voltage            (N+4)x0.062   V

2C  JTA output power                  2.0x(N+4)^1.618mW

2D  calibration voltage               (N+4)/50      V

3A  battery temperature               1.4x(67-N)  deg. C

3B  baseplate temperature #1          1.4x(67-N)  deg. C

3C  baseplate temperature #2          1.4x(67-N)  deg. C

3D  baseplate temperature #3          1.4x(67-N)  deg. C

The  status  byte conversions are  shown  below.  This method  is

used because all items whose status is represented in this manner

only  have  two possible situations, either ON or OFF, or  binary

values  0 or 1. For example, if the first item of status 4A  were

423,   the 4 identifies group 4, and the 23 should be thought  of

as its equivalent binary  code (10011).  This shows the status in

the  order of MSB to LSB, or bit 4 to bit 0. Using  the  decoding

data 423 can be decoded as follows.

1: Beacon is PSK,

0: Engineering data #2 is blank,

0: Engineering  data  #1 is blank,

1: JTD power is ON,

1: JTA power is ON.

------------------------------------------------------

Mode JA System Status Bytes

CH BIT  DESCRIPTION     STATE

1     0

=========================================================

4A  0   JTA power       ON    OFF

4A  1   JTD power       ON    OFF

4A  2   Eng. data #1    ---   ---

4A  3   Eng. data #3    ---   ---

4A  4   Beacon          PSK   CW

4B  0   UVC             ON    OFF

4B  1   UVC level       1     2

4B  2   Battery         tric  full

4B  3   Battery logic   tric  full

4B  4   Main relay      ON    OFF

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX B The Fuji-OSCAR 20 Spacecraft Page 37

4C  0   PCU             bit 1 (LSB)

4C  1   PCU             bit 2 (LSB)

4C  2   PCU             manual  auto

4C  3   Eng. data #3    ---   ---

4C  4   Eng. data #4    ---   ---

4D  0   Memory bank #0  ON    OFF

4D  1   Memory bank #1  ON    OFF

4D  2   Memory bank #2  ON    OFF

4D  3   Memory bank #3  ON    OFF

4D  4   Computer power  ON    OFF

5A  0   Memory select   bit 1 (LSB)

5A  1   Memory select   bit 2 (MSB)

5A  2   Eng. data #5    ---   ---

5A  3   Eng. data #6    ---   ---

5A  4   Eng. data #7    ---   ---

5B  0   Solar panel #1  lit   dark

5B  1   Solar panel #2  lit   dark

5B  2   Solar panel #3  lit   dark

5B  3   Solar panel #4  lit   dark

5B  4   Solar panel #5  lit   dark

5C  0   JTA CW beacon   CPU   TLM

5C  1   Eng. data #8    ---   ---

5C  2   Eng. data #9    ---   ---

5C  3   Eng. data #10   ---   ---

5C  4   Eng. data #11   ---   ---

5D  0   Eng. data #12   ---   ---

5D  1   Eng. data #13   ---   ---

5D  2   Eng. data #14   ---   ---

5D  3   Eng. data #15   ---   ---

5D  4   Eng. data #16   ---   ---

Mode JD Telemetry Data

Telemetry    data  are  also  sent  on  Mode  JD  by   means   of

packets.These  data  are  transmitted the ASCII format  shown  in

Table  4.  In the ASCII telemetry  (RA and RB) XXX  is a 3  digit

decimal  number  with a a range between 000 to 999.  This  number

represents  the value of N in Table 5 for channels denoted #00  -

#26.

Table  5 contains the equations for converting the received  data

into  engineering  values.  The YYY bytes are  three  hexadecimal

bytes  of  system  status data, denoted #27a - #29c  and  can  be

decoded  as  shown in Table 6. The SSS byte in the last  row  are

binary  status data, denoted #30a - #39c.  Table  7 provides  the

information needed to decode them in a manner similar to the Mode

JA status points shown in Table 3.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX B The Fuji-OSCAR 20 Spacecraft Page 38

Table 4.  Mode JD PSK telemetry data format

JAS-1b FF YY/MM/DD HH:MM:SS

XXX XXX XXX XXX XXX XXX XXX XXX XXX XXX

XXX XXX XXX XXX XXX XXX XXX XXX XXX XXX

XXX XXX XXX XXX XXX XXX XXX YYY YYY YYY

SSS SSS SSS SSS SSS SSS SSS SSS SSS SSS

where,   FF  is  the  Frame Identifier,  which  may  contain  the

following types:

RA: Realtime telemetry, - ASCII

RB: Realtime telemetry, - Binary

SA: Stored telemetry, - ASCII

SB: Stored telemetry, - Binary

M0: Message #0

M1: Message #1

...........

M9: Message #9

YY/MM/DD is year/month/day, and HH:MM:SS is hour/minute/second,

all in UTC.

Table 5.  Mode JD Telemetry Decoding Equations

CH   DESCRIPTION                  CALIBRATION

=========================================================

#00  total solar array current     1.91x(N-4)mA

#01  battery charge/discharge     -3.81x(N-508)mA

#02  battery voltage              Nx0.022V

#03  battery center voltage       Nx0.009961V

#04  bus voltage                  Nx0.02021 V

#05  +5 V regulator voltage       Nx0.00620 V

#06  -5 V regulator voltage      -Nx0.00620 V

#07  + 10 V regulator voltage     Nx0.0126  V

#08  JTA output power             5.1x(N-158)mW

#09  JTD output power             5.4x(N-116)mW

#10  calibration voltage #2       N/500 V

#11  offset voltage #1            N/500 V

#12  battery temperature          0.139x(669-N)deg. C

#13  JTD temperature              0.139x(669-N)deg. C

#14  Baseplate Temperature #1     0.139x(669-N)deg. C

#15  Baseplate Temperature #2     0.139x(669-N)deg. C

#16  Baseplate Temperature #3     0.139x(669-N)deg. C

#17  Baseplate Temperature #4     0.139x(669-N)deg. C

#18  temperature calibration #1   N/500 V

#19  offset voltage #2            N/500 V

#20  Solar Cell Panel Temp   #1   0.38x(N-685)deg. C

#21  Solar Cell Panel Temp   #2   0.38x(N-643)

#22  Solar Cell Panel Temp   #3   0.38x(N-646)

#23  Solar Cell Panel Temp   #4   0.38x(N-647)

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX B The Fuji-OSCAR 20 Spacecraft Page 39

#24  -------------------------

#25  temperature calibration #2   N/500 V

#26  temperature calibration #3   N/500 V

---------------------------------------------------------

Table 6.  Mode JD HEX System Status Bytes

CH   DESCRIPTION

=========================================================

#27a  Spare (TBD)

#27b  Spare (TBD)

#27c  Spare (TBD)

#28a  Spare (TBD)

#28b  Spare (TBD)

#28c  error count of memory unit #0

#29a  error count of memory unit #1

#29b  error count of memory unit #2

#29c  error count of memory unit #3

----------------------------------------------

Table 7.  Mode JD BINARY System Status Bytes.

CH    DESCRIPTION             STATE

1      0

=========================================

#30a  JTA power             on     off

#30b  JTD power             on     off

#30c  JTA beacon            PSK    CW

#31a  UVC status            on     off

#31b  UVC level             1      2

#31c  main relay            on     off

#32a  engineering data #1     -----

#32b  battery status        tric   full

#32c  battery logic         tric   full

#33a  engineering data #2     -----

#33b  PCU status            bit 1 (LSB)

#33c  PCU status            bit 2 (MSB)

#34a  memory unit  #0       on    off

#34b  memory unit  #1       on    off

#34c  memory unit  #2       on    off

#35a  memory unit           on     off

#35b  memory select         bit 1 (LSB)

#35c  memory select         bit 2 (MSB)

#36a  engineering data #3     ------

#36b  engineering data #4     ------

#36c  computer power        on    off

#37a  engineering data #5     ------

#37b  solar panel #1        lit  dark

#37c  solar panel #2        lit  dark

#38a  solar panel #3        lit  dark

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX B The Fuji-OSCAR 20 Spacecraft Page 40

#38b  solar panel #4        lit  dark

#38c  solar panel #5        lit  dark

#39a  engineering data #6     ------

#39b  CW beacon source      CPU   TLM

#39c  engineering data #7     ------

Summary

This  article  has  been  an introduction  to  Fuji-OSCAR  20,  a

friendly  little bird which provides strong mode J  signals  both

for analog and digital communications as well as telemetry  which

can  be  used for educational purposes.  If you can work  mode  B

then you ought to be able to work mode J with just a little  more

effort.  Tune in one evening and say 'Kon bon wa' as it goes  by.

Remember the 'J' in mode 'J' stands for Japan.

Acknowledgments

This  article  has been compiled from information  received  from

AMSAT-UK,  DB0OS,  KI6QL,  JAMSAT and the JARRL.   It  was  first

published in the AMSAT Journal, Volume 13, Numbers 3 and 4,  July

and September 1990.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHAT-UP APPENDIX C Fuji-OSCAR 12 Telemetry Page 41

This  Appendix contains information for decoding the data in  the

Fuji-OSCAR  12  PSK  Telemetry Data  Format.  It  is  practically

identical to the Fuji-OSCAR 20 format.

JAS-1 FF YY/MM/DD HH:MM:SS

xxx xxx xxx xxx xxx xxx xxx xxx xxx xxx

xxx xxx xxx xxx xxx xxx xxx xxx xxx xxx

xxx xxx xxx xxx xxx xxx xxx xxx yyy yyy

sss sss sss sss sss sss sss sss sss sss

FF := Frame Identifier   RA: Realtime Telemetry - ASCII

RB: Realtime Telemetry - Binary

SA: Stored Telemetry - ASCII

SB: Stored Telemetry - Binary

M0: Message #0

M1: Message #1

.......

M9: Message #9

YY/MM/DD  =  Date

HH:MM:SS  =  Time (The command station attempts to keep the clock

as

close as possible to UTC)

[ Following is valid only for RA and SA frames ]

xxx =  000 - 999  Format: 3 digit decimal (Analog Data)

28  samples  in  row 0 column 0   thru   row  2

column 7

(denoted #00 - #27 below)

y =  0 - F      one byte Hex (System Status Data)

9 samples in row 2 column 8  thru  row 2 column

9

(denoted #28a - #29c below)

s =  0 or 1     Binary Status Data

30 samples in row 3  thru  row 3 column 9

(denoted #30a - #39c below)

***  FO-12 Telemetry Calibration Equations  ***

Ch.     Item                          Equation

--------------------------------------------------------------------

#00     Total Solar Array Current     1.91 * ( N - 4 )    mA

#01     Battery Charge/Discharge      3.81 * ( N - 528 )  mA

#02     Battery Voltage               N * 0.0210    V

#03     Half-Battery Voltage          N * 0.00937   V

#04     Bus Voltage                   N * 0.0192    V

#05     + 5 V. Regulator Voltage      N * 0.00572   V

#06     - 5 V. Regulator Voltage      N * -0.00572  V

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHAT-UP APPENDIX C Fuji-OSCAR 12 Telemetry Page 42

#07     +10 V. Regulator Voltage      N * 0.0116    V

#08     JTA Power Output              5.1 * ( N  - 158 )  mW

#09     JTD Power Output              5.4 * ( N - 116 )   mW

#10     Calibration Voltage #2        N / 500  V

#11     Offset Voltage #1             N / 500  V

#12     Battery Temperature           0.139 * ( 689 - N )  Deg. C

#13     JTD Temperature               0.139 * ( 689 - N )  Deg. C

#14     Baseplate Temperature #1      0.139 * ( 689 - N )  Deg. C

#15     Baseplate Temperature #2      0.139 * ( 689 - N )  Deg. C

#16     Baseplate Temperature #3      0.139 * ( 689 - N )  Deg. C

#17     Baseplate Temperature #4      0.139 * ( 689 - N )  Deg. C

#18     Temperature Calibration #1    N / 500  V

#19     Offset Voltage #2             N / 500  V

#20     Facet Temperature #1          0.38 * ( N - 684 )  Deg. C

#21     Facet Temperature #2          0.38 * ( N - 684 )  Deg. C

#22     Facet Temperature #3          0.38 * ( N - 690 )  Deg. C

#23     Facet Temperature #4          0.38 * ( N - 683 )  Deg. C

#24     Facet Temperature #5          0.38 * ( N - 689 )  Deg. C

#25     Temperature Calibration #2    N / 500  V

#26     Temperature Calibration #3    N / 500  V

#27     Depth of Battery discharge    ( N - 500 ) / 189   AH

***  FO-12 System Status Telemetry Bytes ***

Ch.     Item

--------------------------------------------------

#28a    Spare (TBD)

#28b    Spare (TBD)

#28c    Memory Unit #0 error count

#29a    Memory Unit #1 error count

#29b    Memory Unit #2 error count

#29c    Memory Unit #3 error count

***  FO-12 Binary Status Data Points ***

Ch.    Item                     1         0

-----------------------------------------------

#30a   JTA Power                On        Off

#30b   JTD Power                On        Off

#30c   JTA Beacon               PSK       CW

#31a   UVC Status               On        Off

#31b   UVC Level                1         2

#31c   Main Relay               On        Off

#32a   Engineering Data #1      ----      ----

#32b   Battery Status           Tric      Full

#32c   Battery Logic            Tric      Full

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHAT-UP APPENDIX C Fuji-OSCAR 12 Telemetry Page 43

#33a   Engineering Data #2      ----      ----

#33b   PCU Status                Bit 1 (LSB)

#33c   PCU Status                Bit 2 (MSB)

#34a   Memory Unit #0           On        Off

#34b   Memory Unit #1           On        Off

#34c   Memory Unit #2           On        Off

#35a   Memory Unit #3           On        Off

#35b   Memory Select             Bit 1 (LSB)

#35c   Memory Select             Bit 2 (MSB)

#36a   Engineering Data #3      ----      ----

#36b   Engineering Data #4      ----      ----

#36c   Computer Power           On        Off

#37a   Engineering Data #5      ----      ----

#37b   Solar panel #1           Lit       Dark

#37c   Solar panel #2           Lit       Dark

#38a   Solar panel #3           Lit       Dark

#38b   Solar panel #4           Lit       Dark

#38c   Solar panel #5           Lit       Dark

#39a   Engineering Data #6      ----      ----

#39b   CW beacon source         CPU       TLM

#39c   Engineering Data #7      ----      ----

***  Example  ***

JAS-1 RA 86/08/01 09:00:00

500 xxx xxx xxx xxx xxx xxx xxx xxx xxx

xxx xxx xxx xxx xxx xxx xxx xxx xxx xxx

xxx xxx xxx xxx xxx xxx xxx xxx 004 yyy

01s sss sss sss sss sss sss sss sss sss

Real time ASCII frame sent on 86/08/01 at 09:00:00 UTC

Total Solar Array Current  =  947 mA

Memory Unit #0 error count =  4

JTA (mode-JA TX) power Off

JTD (mode-JD TX) power On

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHAT-UP APPENDIX C Fuji-OSCAR 12 Telemetry Page 44

APPENDIX D Information about AMSAT

For  further  information  about  the  Radio  Amateur   Satellite

program,  photocopy and mail the following form together  with  a

self addressed stamped envelope (SASE) to :-

To:

[ ]       AMSAT-NA,  850 Sligo Avenue, Silver Spring, MD,  20910-

4703. Telephone (301) 589 6062.

[ ]       Project OSCAR Inc. POB 1136, Los Altos, CA. 94023-1136.

[ ]       AMSAT-UK, 94 Herongate Road, Wanstead Park, London  E12

5EQ.  Telephone (081) 989 6741.

Please  send  me  further information  about  the  Radio  Amateur

Satellite   program,   and   details  of   membership   in   your

organization.

CALL ________________ TODAY'S DATE _____________

NAME ______________________________________________

ADDRESS ___________________________________________

___________________________________________

CITY ___________________________________________

STATE _________ POSTCODE ______________________

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHAT-UP APPENDIX C Fuji-OSCAR 12 Telemetry Page 45

APPENDIX E Amateur Radio Software by Joe Kasser G3ZCZ

PC-HAM 3.3

PC-HAM contains a number of programs some of which are  described

below.

LOGBOOK

Full  blown  logging package. With automatic check  of  logs  for

awards such as DXCC. Allows you to recall any entry by call  sign

within  seconds.  Indexed displays, QSLing, Contest  mode  QSLing

(prints the lot) and lots more written in dBASE3, but a  compiled

LOGBOOK.EXE file is supplied together with the source code.

CONTEST

Keeps Dupes in memory, logs QSO's to disk in format which can be

processed by the LOGBOOK package.

CQSS

Sweepstakes  game.  Work  the ARRL Sweepstakes  contest  on  your

computer. You are located just outside Washington DC. A  propaga-

tion  model is built in to the program. This program is  REQUIRED

training  for all sweepstakes operators. Program is based on  the

one  described in detail in 'Software for Amateur Radio'  by  Joe

Kasser  G3ZCZ,  published by TAB Books, Blue  Ridge  Summit,  PA.

17214.

WHATSON

Predict HF Propagation. Contest mode with printout to whole world

at hourly intervals. Needs BASIC

LAN-LINK 1.58

Function  key  and  Menu driven. Automatic  logbook  entries  for

Packet  and  Mailbox/Beacon Mode  AMTOR  Connects,  semiautomatic

logbook entries for other modes. Log file can be processed by the

Logbook Package of PC-HAM.

Contest  operation,  sends  standard  message  and  automatically

increments  QSO count. Automatic optimized configuration  of  the

TNC  for each communications mode. All mode Function  key  'OVER'

feature (End).

There  are 10 files with fixed names (LAN-LINK.001  through  LAN-

LINK.010)  which may be viewed and transmitted by means of  func-

tion keys. They may also be edited from the Edit Menu.

Set up of TNC for AMSAT-OSCAR (non packet) Telemetry reception.

Time display and event scheduler. ASCII Text Editor. Customizable

Colors.  Access to the TNC Command Mode is provided in  case  the

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHAT-UP APPENDIX C Fuji-OSCAR 12 Telemetry Page 46

user wishes to override any defaults.

Automatic capture to disk of all packet radio connects. Automatic

indication  of the number of Packet connects. Local Area  Network

(LAN) message store and Forward capability.

Capable  of  automatic connect attempts to download  a  QTC  from

another station in the LAN. Capable of automatic connect attempts

to  a  packet BBS to download your incoming messages,  when  your

callsign  appears on the BBS mail beacon annunciator. Capable  of

automatically requesting Bulletins on subjects that interest  you

from your local packet BBS.

Digipeat  monitoring  and capture. Alert signal to let  you  know

when  a  predetermined  call  shows up  in  a  packet  header  on

frequency.

Conference  Mode  in  multiconnect  situations.  Bridge  Mode  in

multiconnect  situations.  Path determination to DX  station  via

:QMH:.  Indicator  that  a specific  station  designated  as  the

'target'  call  connected  in Packet Mode,  or  linked  to  AMTOR

Beacon/Mailbox while you were away.

Automatic  NET/ROM  and  KA Node path set  up  from  LAN-LINK.DIR

call/path directory file. Selective answering machine and MAILBOX

using  NC/L  command dialogue. Screen indication  of  connect  by

desired station (target call).

Automatic  Beacon Mode CQ caller. Will call CQ  repetitively  and

either work the connect and keep going after disconnect or signal

you when a reply is received.

SAREX special features. :QRA: trigger to determine who else is on

channel.

Automatic  AMTOR SELCAL determination. Function key  change  from

monitoring AMTOR FEC CQ's to QSO's in progress (chirpcopy).

NAVY MARS RTTY file transmitting protocols for the PK-232.

Log files in dBASE 3 format.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX F SHAREWARE Page 47

The  Association of Shareware Professionals (ASP) has established

standards for its members and for any organization which has "ASP

Approved"  status. The ASP wants to make sure that the  shareware

principle  works for you. If you are unable to resolve a  problem

with  an  ASP  member  or  organization  (other   than  technical

support),  the ASP may be able to help. Please write to

The  ASP Ombudsman, P.O. Box 5786, Bellevue, WA  98006, USA.

You  are encouraged to copy the floppy disk and share  it  freely

with  others.  You have the luxury of trying out the  product  at

your own pace and in the comfort of your own home or workplace.

After  you  have used the material for  a  reasonable  evaluation

period  (90  days),  you should either  discontinue  use  of  the

material  or register your copy.  Your support is  important  and

greatly  appreciated.  With it, Shareware authors are  encouraged

to  design and distribute new products. Without it, a great  deal

of high quality, low cost software will cease to be available.

Why pay at all?

*  You receive support from the author.

*  You receive a CURRENT copy of the program.

*  Your input and ideas help shape future products.

*  A sense of pride and ownership in having honestly

participated in the Shareware revolution.

*  You help to keep software prices down by supporting a

distribution method which doesn't depend on expensive

advertising campaigns.

Be  aware of the following restrictions, designed to protect  the

community  of Shareware users and to prevent greedy  people  from

taking unfair advantage of the trust, hard work and good will  of

Shareware authors.

1.   No  price or consideration may be charged for the  material.

However, a distribution cost may be charged for the cost  of

the diskettes, shipping and handling, not to exceed $6.

2.   The  files and programs on the disks may not be modified  or

deleted.

3.   The  material  cannot  be sold as part of  some  other  more

inclusive package.

4.   The material cannot be "rented" or "leased" to others.

5.   The end user must be told clearly in writing on the  outside

of  the package and in all advertising that the  diskette(s)

are "Shareware."

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX F SHAREWARE Page 48

6.   The package must contain a written explanation that the disk

is   for  evaluation  purposes,  and  that   an   additional

"registration  fee"  is  expected  by  the  author,  if  the

material is used beyond an initial evaluation period.

7.   In the case of distribution via any telecommunications link,

the following must be done:

An error checking protocol must be used.

The individual files must be combined into, and  transferred

in a library or archive format.

8.   Shareware  distribution  is  permitted only  in  the  United

States, Canada, England, and Australia.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP APPENDIX F SHAREWARE Page 49

WHATS-UP 0.55 Registration Fee $35.00

LAN-LINK 1.58 Registration Fee $35.00

PC-HAM   3.52 Registration Fee $36.50

Register them all for $95.00.

Evaluation Copies free, you pay only for the disk/postage. For an

evaluation  copy  (unregistered  full  blown  version)  of   each

software  send a formatted disk and SASE to Joe Kasser.  Overseas

users,  send  $5.00 ($10.00 covers all three)  instead.  Try  the

software  first, and only register your copy if you make  use  of

it.

To: Joe Kasser G3ZCZ,  P O BOX 3419,  SILVER SPRING,  MD 20918.

CALL ________________ TODAY'S DATE _____________

NAME ______________________________________________

ADDRESS ___________________________________________

___________________________________________

CITY ___________________________________________

STATE _________ POSTCODE ______________________

TNC TYPE _____________________ Home BBS __________

I enclose a check for ______. Please send me the latest version

of ______________, and register me as a user.

I enclose a disk/sase or $5.00. Please send me the latest version

of ______________________ to evaluate. If I like it and use it, I

plan to become a registered user.

Disk Size 5.25 _____(360k) 5.25 _____(1.2Mb) 3.5 ______(730k)

I AM NOT/AM currently using Shareware Version _____

which I obtained from _________________________________________.

Signature _______________________

Radio Club Registrations 10 or more copies, 10% discount.

50 or more copies, 25% discount.

Designate one contact person to receive club registered  updates,

and  supply  a list of names and call signs of club  members  who

will be using the Registered Club Copy.

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP Index Page 50

Accuracy, 19

Alarm, 13, 14

Algebra, 18

Alt-B, 6

Alt-C, 6

Alt-D, 6

Alt-P, 6

Alt-S, 6

Alt-X, 6

Altitude, 14

AMSAT, 1, 2, 3, 4, 7, 9, 10, 12, 18, 20, 21, 23, 40, 44

AMSAT-NA, 18, 44

AMSAT-OSCAR, 7, 15, 45

AMSAT-UK, 40, 44

Analog-to-digital, 19

Analysis, 1, 4, 6

Analyze, 7

Annunciator, 46

Antenna, 32

Antennas, 32

Apogee, 31

Archive, 48

Array, 6, 17, 22, 23, 24, 25, 26, 27, 28, 29, 30, 35, 36, 38, 41,

43

ARRAYS, 14

BCR, 24, 26, 27, 28, 30

BCRXMT, 5

BCXRT, 20

Beacon, 4, 9, 10, 16, 17, 32, 33, 34, 35, 36, 37, 39, 40, 42, 43,

45, 46

Capture-to-disk, 4, 12, 13, 15

Channel, 3, 4, 6, 12, 14, 16, 17, 18, 19, 20, 21, 30, 32, 35, 46

Codes, 6

Coefficients, 7, 18, 23, 25, 27, 29

Color, 2, 5, 6, 9, 11, 13, 14, 15, 16, 20, 21

Column, 6, 19, 21, 41

Combinations, 2, 6, 11, 35

Configuration, 1, 2, 5, 7, 14, 16, 45

Contains, 4, 16, 17, 18, 20, 22, 35, 37, 41, 45

Contents, 2, 5, 6, 11, 14, 15, 21

Copy, 1, 4, 7, 9, 10, 12, 32, 47, 49

Correct, 6, 7, 8, 16

Count, 5, 21, 23, 25, 27, 29, 39, 42, 43, 45

Current, 23, 25, 27, 29, 35, 36, 38, 41, 43, 47

Decimal, 5, 16, 18, 19, 20, 35, 37, 41

Decode, 1, 7, 17, 35, 37

Default, 5, 6, 8, 10, 11, 12, 13, 14, 15, 17, 21

Deg, 23, 24, 25, 26, 27, 28, 29, 30, 36, 38, 42

DOVE, 1, 3, 8, 10, 14, 15, 16, 17, 25

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP Index Page 51

DOVE-OSCAR, 5, 15

Engineering, 4, 5, 6, 13, 19, 35, 36, 37, 39, 40, 42, 43

Envelope, 44

Equations, 4, 18, 23, 24, 25, 26, 27, 28, 29, 30, 34, 36, 37, 38,

41

Example, 5, 7, 10, 15, 16, 19, 21, 35, 36, 43

Exercise, 9

Extract, 1, 2, 6, 9, 14, 33

File, 2, 3, 4, 6, 7, 8, 9, 10, 11, 13, 14, 15, 16, 18, 20, 21, 22,

45, 46

Files, 1, 2, 5, 7, 9, 10, 11, 12, 13, 15, 45, 46, 47, 48

First, 1, 2, 7, 12, 13, 15, 17, 18, 20, 21, 31, 33, 36, 40, 49

Format, 2, 4, 5, 6, 7, 10, 16, 18, 20, 34, 35, 37, 38, 41, 45, 46,

48

Fuji, 8, 9, 10, 15, 16, 18, 20, 31, 33

Fuji-OSCAR, 1, 3, 7, 15, 18, 31, 32, 33, 34, 35, 40, 41

Header, 6, 21, 46

HEADERLINE, 6, 12

Hexadecimal, 16, 20, 37

Identifier, 16, 17, 18, 19, 38, 41

Institution, 1

Interactive, 2, 4, 6, 7, 9, 12

LAN-LINK, 5, 12, 45, 46, 49

Limit, 2, 5, 13, 14, 16, 19, 20

Lines, 14, 16, 20, 21

Link, 2, 4, 5, 7, 20, 21, 32, 48

Menu, 2, 6, 8, 9, 11, 12, 13, 45

Microsat, 2, 7, 8, 9, 10, 17, 18, 20, 23

Non-existent, 13

Options, 9, 11, 12, 13

OSCAR, 1, 2, 4, 9, 12, 32, 33, 44

Output, 24, 26, 28, 30, 32, 36, 38, 42

Packet, 5, 6, 7, 9, 10, 11, 12, 13, 15, 20, 21, 32, 34, 35, 45, 46

Protocol, 32, 48

Protocols, 46

Quadratic, 18

Real-time, 4, 7, 10, 13, 15

Satellite, 4, 31, 32, 34, 44

Self, 44

Simultaneous, 34

COPYRIGHT Joe Kasser, G3ZCZ 1990.

WHATS-UP Index Page 52

Spreadsheet, 4, 6, 13, 22

Stamped, 44

Status, 4, 5, 14, 34, 35, 36, 37, 39, 41, 42, 43, 47

Telemetry, 1, 3, 4, 5, 6, 7, 8, 9, 10, 11, 14, 16, 17, 18, 19, 21,

22, 23, 25, 27, 29, 32, 33, 34, 35, 36, 37, 38, 40, 41, 42, 45

WHATS-UP, 1, 2, 4, 5, 6, 7, 8, 10, 11, 13, 14, 15, 16, 17, 18, 19,

20, 21, 49

COPYRIGHT Joe Kasser, G3ZCZ 1990.

########################################################################################################################################################################################################################################################